# Multimodal Data Analysis for OCD Patients Under Deep Brain Stimulation

GitHub Repository Documentation

Agustina Amilibia | Rice University | Fall 2025

# About This Project

This repository contains the code developed for my Master's Capstone Project in Electrical and Computer Engineering at Rice University, in collaboration with the Provenza Lab at Baylor College of Medicine.

The project aims to develop tools for analyzing multimodal data from patients with treatment-resistant OCD undergoing Deep Brain Stimulation (DBS). The ultimate goal is to identify neural biomarkers that could enable closed-loop, adaptive DBS systems that respond to patient anxiety states in real-time.

## Research Context

Obsessive-Compulsive Disorder (OCD) affects millions worldwide, and for some patients, traditional treatments are ineffective. Deep Brain Stimulation offers hope, but current systems deliver constant stimulation 24/7—regardless of the patient's actual needs.

Our research works toward closed-loop DBS: a smarter system that monitors brain signals and adjusts stimulation automatically based on detected anxiety levels. This requires identifying reliable biomarkers—measurable brain signals that correlate with anxiety states.

# Table of Contents

- Repository Structure

- Project Components

- Quick Start Guide

- Key Findings

- Data Format Requirements

- Experimental Protocol

- Future Directions

- Acknowledgments

# 1. Repository Structure

| Directory/File | Description |
| --- | --- |
| audio_video_sync/ | Stage 1: Audio-Video Synchronization (Arduino + MATLAB) |
| video_lfp_pipeline/ | Stage 2: Real-Time Multimodal Pipeline (MATLAB) |
| lfp_analysis/ | Stages 3-5: Neural Analysis & ML (Python) |
| classes/LFPData.py | Data loading and preprocessing class |
| classes/BandAnalysis.py | Frequency band power analysis class |
| plot_functions.py | Visualization utilities |
| analysis.ipynb | Main analysis notebook |

# 2. Project Components

## 2.1 Audio-Video Synchronization System

Language: Arduino + MATLAB

A custom synchronization system that enables frame-accurate alignment between audio and video recordings during clinical sessions. Uses an Arduino-based serial encoder that embeds timing markers into both audio and video streams.

Key Features:

- Custom PCB design for signal routing

- Serial frame ID encoding in audio channel

- Automated decoding and alignment in MATLAB

- Sub-frame timing accuracy

## 2.2 Video-LFP Synchronization Pipeline

Language: MATLAB

Creates synchronized visualization videos that display clinical recordings alongside real-time neural data (LFPs, theta oscillations, and spectrograms). Essential for correlating patient behavior with neural activity.

Key Features:

- Real-time synchronized playback of video + neural data

- Bilateral LFP signal visualization

- Theta band (4-8 Hz) extraction

- Live spectrogram display (0-30 Hz)

- FFmpeg audio integration

## 2.3 LFP Analysis Pipeline (Python)

Language: Python

Object-oriented framework for analyzing Local Field Potentials and their relationship with anxiety states (measured via SUDS - Subjective Units of Distress Scale).

Key Features:

- Modular OOP architecture for scalability

- Automated preprocessing (NaN interpolation, demeaning, artifact removal)

- Band power calculation (Theta, Alpha, Beta)

- SUDS-LFP alignment and correlation analysis

- Machine learning classification (Logistic Regression, Random Forest)

# 3. Quick Start Guide

## Prerequisites

For MATLAB components:

- MATLAB R2020a or later

- Signal Processing Toolbox

- FFmpeg (for audio processing)

For Python components:

pip install numpy pandas scipy matplotlib seaborn plotly scikit-learn

## Running the Python Analysis

from classes import LFPData, BandAnalysis

# Load and preprocess patient data

patient = LFPData("AA002", "2025-04-10")

patient.load_data(lfp_csv_path, task_csv_path)

patient.preprocess(artifact_start_sec=10, artifact_end_sec=5)

# Analyze frequency bands

analysis = BandAnalysis(patient, window_sec=2.0)

analysis.calculate_all_bands()

analysis.print_summary()

# 4. Key Findings

Our preliminary analysis of a single patient revealed:

| Finding | Details |
| --- | --- |
| Biomarker Candidate | Left-hemisphere Theta power (4-8 Hz) shows significant negative correlation with anxiety (SUDS) |
| Phase Discrimination | Relief phase shows highest Theta power; Exposure phase shows lowest |
| ML Performance | Phase-based stress detection: 61.5% accuracy (above 50% chance level) |
| Feature Importance | Theta_left is the most predictive feature (importance = 0.22) |

These findings suggest that left-lateralized Theta activity may serve as a potential biomarker for anxiety states in OCD patients with DBS.

# 5. Data Format Requirements

## Neural Data (.mat file)

Expected structure from Percept PC recordings:

- data.neural.combined_data_table - LFP channels with timestamps

- data.neural.fs - Sampling frequency (typically 250 Hz)

- data.behavior.events - Event markers including sync beep (Event Code 10)

## Task Data (.csv file)

Required columns:

- Timer - Format MM:SS:ms

- Event - Contains 'SUDS' and 'Note' entries

- Entry - Phase markers: 'Task Start', 'exposure', 'compulsions', 'relief'

# 6. Experimental Protocol

The ERP (Exposure and Response Prevention) task consists of four phases:

| Phase | Duration | Description |
| --- | --- | --- |
| Baseline | 2 min | Rest period, calibration |
| Exposure | 6 min | Patient exposed to anxiety trigger |
| Compulsions | 4 min | Patient performs compulsive behaviors |
| Relief | 4 min | Recovery period |

SUDS ratings (0-10) are collected throughout the session via tablet.

# 7. Future Directions

- Scale to More Patients - Generalize the OOP pipeline across multiple subjects

- Richer Neural Features - Add bursting dynamics, band ratios, asymmetry indices

- Multimodal Integration - Incorporate video-based facial affect and audio features

- Biomarker Validation - Identify consistent neural signatures for closed-loop DBS

# 8. Publications & Presentations

- WSSFN 2025 (Buenos Aires, Argentina) - Poster: "DBS Data Synchronization with Continuous Audio and Video for Psychiatric Biomarker Discovery"

# 9. Acknowledgments

Provenza Lab, Baylor College of Medicine:

- Dr. Nicole Provenza - Principal Investigator

- Tori Gobo - PhD Student

- Raphael Bechtold - PhD Student

- Yewen Zhou - Software Engineering Associate

Rice University ECE Department - Capstone Program Support

# Author

Agustina Amilibia

Master's Student, Electrical and Computer Engineering

Rice University

Fall 2025