# Video-LFP Synchronization Tool

This MATLAB script creates synchronized videos that display clinical video recordings alongside real-time LFP (Local Field Potential) neural data visualizations. The tool is designed for ERP (Exposure and Response Prevention) analysis in DBS-OCD patients, allowing researchers to observe behavioral and neural correlates simultaneously.

# Table of Contents

- Overview and Purpose

- Requirements

- Input Files

- Script Workflow

- Output Video Layout

- Configuration Parameters

- Usage Instructions

# 1. Overview and Purpose

The script generates a composite video that synchronizes clinical video recordings with neural signal visualizations, enabling frame-by-frame analysis of behavioral-neural correlations during ERP sessions.

## Key Features

- Automatic neural-video alignment using beep marker (Event Code 10)

- Real-time display of raw LFP signals from both hemispheres

- Theta band (4-8 Hz) filtered signal visualization

- Live spectrograms showing frequency content (0-30 Hz)

- Clock time and elapsed time overlays

- Audio track preservation from original video

# 2. Requirements

## Software

- MATLAB (R2020a or later recommended)

- Signal Processing Toolbox (for bandpass filtering)

- FFmpeg (for audio merging)

## FFmpeg Installation

The script requires FFmpeg for adding the audio track to the final video. The default path is configured for macOS Homebrew installation:

ffmpeg_path = '/opt/homebrew/bin/ffmpeg';

Installation options:

- macOS: brew install ffmpeg

- Windows: Download from ffmpeg.org and update path

- Linux: sudo apt install ffmpeg

# 3. Input Files

## 3.1 Neural Data (.mat file)

The script expects a preprocessed .mat file containing the following structure:

| Field | Description |
| --- | --- |
| data.neural.combined_data_table | Table with LFP channels, Timestamp, and Datetime columns |
| data.neural.fs | Sampling frequency (typically 250 Hz) |
| data.behavior.events | Event table with Timestamp and Event_Code columns |
| data.behavior.first_event_unix_time | Unix timestamp for task start reference |

### Supported LFP Channel Names

The script automatically detects one of the following channel naming conventions:

- TD_Aic_ONE_THREE_LEFT / TD_Aic_ONE_THREE_RIGHT

- TD_Aic_ZERO_TWO_LEFT / TD_Aic_ZERO_TWO_RIGHT

- TD_Other_ZERO_TWO_LEFT / TD_Other_ZERO_TWO_RIGHT

- TD_Other_ONE_THREE_LEFT / TD_Other_ONE_THREE_RIGHT

## 3.2 Video File (.mp4)

Clinical video recording in MP4 format. The video must contain an audio track with a synchronization beep that corresponds to Event Code 10 in the neural data.

# 4. Script Workflow

## Processing Steps

- Load Neural Data: User selects .mat file via file dialog

- Extract Subject ID: Parses filename to create output folder name

- Synchronization: Finds Event Code 10 (beep) timestamp and aligns neural data

- Channel Detection: Automatically identifies LFP channel naming convention

- Preprocessing: Demeans LFP signals and replaces NaN values with zeros

- Theta Filtering: Applies 4-8 Hz bandpass filter to extract theta oscillations

- Spectrogram Computation: Calculates time-frequency representation using short-time FFT

- Video Selection: User selects corresponding MP4 video file

- Frame Animation: Iterates through video frames, updating all plots in sync

- Audio Merge: Uses FFmpeg to combine silent output with original audio

## Signal Processing Details

### Preprocessing

% Demean: remove DC offset

LFP_demeaned = LFP - mean(LFP, 'omitnan');

% Replace NaN with 0

LFP_demeaned(isnan(LFP_demeaned)) = 0;

### Theta Band Extraction

theta = bandpass(LFP_demeaned, [4 8], fs);

### Spectrogram Parameters

| Parameter | Value |
| --- | --- |
| Window | 0.5 seconds × sampling rate (Hamming window) |
| Overlap | 75% of window size |
| NFFT | max(256, next power of 2 after window) |
| Frequency range | 0-30 Hz (displayed) |
| Color scale | 0-30 dB (fixed) |

# 5. Output Video Layout

The output video has a resolution of 1920×1080 pixels with the following panel arrangement:

## Panel Layout (5×2 Grid)

| Row | Left Column | Right Column |
| --- | --- | --- |
| 1-2 | Clinical Video (spans both columns) |  |  |
| 3 | Raw LFP - Left Hemisphere | Raw LFP - Right Hemisphere |
| 4 | Theta Wave - Left (4-8 Hz) | Theta Wave - Right (4-8 Hz) |
| 5 | Spectrogram - Left | Spectrogram - Right |

## Time Display Overlays

- Clock Time: Shows actual datetime (hh:MM:SS PM format)

- Elapsed Time: Shows seconds since video start

## Plot Characteristics

- Time Window: ±5 seconds centered on current frame

- Amplitude Range: ±30 µV for LFP and theta plots

- Current Time Marker: Black vertical line at t=0

- Raw LFP Color: Red

- Theta Band Color: Blue

# 6. Configuration Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| ffmpeg_path | /opt/homebrew/bin/ffmpeg | Path to FFmpeg executable |
| time_window | 5 seconds | Display window (±seconds) |
| max_time | 10 seconds | Testing limit (remove for full video) |
| outputVideo.Quality | 95 | Output video quality (0-100) |
| Figure size | 1920 × 1080 | Output resolution (pixels) |

Note: For full video processing, comment out or remove the max_time condition in the animation loop.

# 7. Usage Instructions

## Step-by-Step Guide

- Configure FFmpeg path: Modify the ffmpeg_path variable if needed

- Run the script: Execute video_and_LFPs_ERP.m in MATLAB

- Select neural data: Choose the preprocessed .mat file when prompted

- Select video: Choose the corresponding .mp4 video file

- Wait for processing: Progress updates will appear in Command Window

- Find output: Final video saved in subfolder named after subject

## Output Files

| File | Description |
| --- | --- |
| {subject}_sync_output.mp4 | Intermediate file (no audio, deleted) |
| {subject}_sync_with_audio.mp4 | Final output with synchronized audio |

## Troubleshooting

- "Could not find LFP channels": Check that .mat file contains expected variable names

- FFmpeg error: Verify FFmpeg is installed and path is correct

- Video/neural mismatch: Ensure Event Code 10 exists and corresponds to audio beep

- Memory issues: Process shorter segments using max_time parameter

# 8. Console Output Example

Using ONE_THREE channels

Time: 0.03s | Progress: 0.0% | Frame: 1

Time: 0.07s | Progress: 0.0% | Frame: 2

...

✅ Video generation complete!

⏳ Adding audio with FFmpeg...

✅ Final video created with audio: /path/to/output.mp4

⏱️ Total elapsed time: 245.3 seconds (4.1 minutes)